## SCHEDULE 6

HEALTH, SAFETY, AND ENVIRONMENTAL PROTECTION

# 1. GENERAL GUIDELINES AND REQUIREMENTS

## 1.1 ENVIRONMENTAL PROTECTION

For each mini-grid project, the potential environmental effects and impacts shall be identified at an early stage of project development

Where the proposed project is likely to have material environmental effects by reason of its nature, scale, or location, an Environmental and Social Impact Assessment (ESIA) shall be undertaken in accordance with section 19(2) of the Mini Grid Regulations 2026.

  - HEALTH AND SAFETY GUIDELINES

To mitigate any adverse effects and impact of its operations and activities on the health and safety of staff members, customers, and other individuals, the mini-grid operator shall maintain a valid insurance policies recommended for mini-grid operations;

# 2. SPECIFIC GUIDELINES AND REQUIREMENTS FOR TYPES OF EQUIPMENT

The table below provides specific requirements regarding the types of equipment that are components in mini-grids –

| TYPE OF EQUIPMENT | PHASE IN THE PROJECT | RECOMMENDED LINK TO ENVIRONMENTAL PROTECTION | HEALTH AND SAFETY |
| --- | --- | --- | --- |
| Battery Valve Regulated Lead Acid (VRLA) | Transport & Storage | - Avoid metals having simultaneous contact with positive and negative terminals of the battery. | - Wear protection gear: shoes, helmet, goggles, and gloves. |
|  | Installation | - Install within bonded walls/containment. - Ensure adequate spacing between batteries to allow for proper cooling. | - Shall only be installed by a competent electrician. - Remove all personal metal objects (wrist watches, rings etc.). - Wear appropriate PPE (eye protection, rubber gloves etc.) - Ensure no smoking near the battery installation. |
|  | Operation | - Keep battery cells neat and clean with soft dry cloth to wipe dirt, oil, moisture, or foreign material. - Do not overcharge the batteries to avoid explosive hydrogen production. - Must be a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Keep a record of all occupational injuries, near-miss and illnesses and divulge these records to the Commission or body on demand for examination and analysis. - Functional risk management process. - All batteries must have readily available safety data sheets. - Should have a comprehensive insurance policy to adequately cover facilities, employees and third parties. |  |
|  | Decommissioning & Disposal |  | - See installation and commissioning. - Batteries must be collected at designated collection points after the end of their lifetime. |
| Battery Lead-Acid Vented | Transport & Storage |  | - Transport battery in upright position and/or transport acid in separate containers. - Acid is a hazardous product: follow the transportation rules for hazardous products. |
|  | Installation |  | - Wear protective gears: shoes, helmet, goggles, and Rubber gloves. - Must only be installed by competent electricians. - Wear an acid-proof protection apron. - During the filling, follow the manufacturers’ recommendations. - Install only in rooms with adequate ventilation. - Avoid any electronics or electric equipment that can generate sparks in the same room. |
|  | Operation | - Keep battery cells clean. | - A comprehensive occupational safety program, including training and site orientation to emergency events. - Competent persons to effectively respond. - Keep a record of all occupational injuries, near-miss and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Do not overcharge the batteries to avoid explosive hydrogen production. - Ensure availability of emergency eye wash station near a battery installation. |
|  | Decommissioning & Disposal |  | - Batteries must be collected at the designated collection points after the end of their lifetime. - Collect acid separately in appropriate canisters prepared for storage and transport |
| Lithium Battery | Installation & Operations |  | - Do not discharge completely and do not overcharge (risk of explosion). - Keep away from solar radiation. |
| Diesel Generator | Transport & Storage | - Generators and oil tanks must be installed within bonded walls/containment. | - Use lifting equipment. - Fuel transport is dangerous and should follow the applicable transport Regulation. - Keep away from flammable materials and sparks. |
|  | Installation |  | - Must only be installed by professional electricians. |
|  | Operation | - Used oil and oil filters shall be collected and disposed of in an environmentally friendly (safe) way. - Select site to meet the following requirements: - Noise Emission should be limited to an extent acceptable by the neighbors. | - Must only be operated on by trained technicians. - Must be in a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Testing and monitoring of air quality is strictly conducted where employees may be exposed to harmful substances, including worker respiratory protection and an emergency response plan. - Competent people to effectively respond to emergency events. - Use safety barricade for work area to prevent unauthorize access. - Workers must wear Appropriate PPE. - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have a comprehensive insurance policy to adequately cover facilities, employees and third parties. - A permit-to-work system shall be strictly observed for all maintenance activities, and all original equipment manufacturer (OEM) recommendations for preventive maintenance and spare parts shall be complied with. |
|  | Decommissioning & Disposal | - Used oil and oil filters shall be collected and disposed of in an environmentally friendly way. - Compliance with Nigerian Electricity Health and Safety Standards Manual Provision in sections. | - Shall be conducted by competent persons. - Use certified lifting equipment for all lifting operations. |
| Solar PV | Operation |  | - Wear safety gear. - Shall be a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have comprehensive insurance policy to adequately cover facilities, employees and third parties. |
|  | Decommissioning & Disposal | - For CdTe modules: Hazardous Waste must be collected and brought to a recycling centre. - Consult manufacturer’s guidelines or where necessary the manufacturer |  |
| Wind Turbine < 50 kW | Transport & Storage |  | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). |
|  | Installation | - Select site to meet the following requirements: - Noise Emission should be limited to an extent acceptable to the neighbours. Shading shall not be more than acceptable to the neighbours. - Must comply with Government laws and Regulations on applicable noise pollution. (Noise standards & control) | - Use appropriate lifting and installation equipment due to the weight of the equipment, (e.g., crane). - By selecting the site, take into consideration the ground characteristics for integrity of the turbine |
|  | Operation | - Switch off wind turbine during main flying hours of bats. - Used oil and oil filters shall be collected and disposed of in an environmentally friendly way. | - Maintain moving parts regularly according to the requirements of the manufacturers. - Must be a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Permit to work (PTW) system including Log Out Tag Out (LOTO) must be followed in all maintenance activities. - Ensure use of appropriate PPE - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have comprehensive insurance policy to adequately cover facilities, employees and third parties. |
|  | Decommissioning & Disposal |  | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). |
| Hydro < 50 kW | Transport & Storage |  | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). |
|  | Installation | - Comply with the water rights and related legislation. | - Moving parts of machinery must be designed and constructed in such a way as to prevent risks of contact which could lead to accidents or must, where risks persist, be fitted with guards or protective devices |
|  | Decommissioning & Disposal | - Comply with the water rights and related legislation. - Used oil and oil filters shall be collected and disposed of in an environmentally friendly way. | - See Transport & Storage |
| Biomass | Transport & Storage | - Sewage from biomass shall be kept from entering the ground water. - Biogas shall be stored in a way to minimize losses to the atmosphere (reduce greenhouse gas emission) | - Depending on the type of biomass, the biomass may carry the risk of fire. Therefore, it shall be stored in a safe fire-protected place. |
|  | Installation |  | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). |
|  | Operation | - Use appropriate Fire protection. - Must be a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have a comprehensive insurance policy to adequately cover facilities, employees and third parties. - A safe system of work must be followed for all Operation and Maintenance activities. |  |
|  | Decommissioning & Disposal | - Any lubrication oil needs to be disposed off in an environmentally friendly manner. - The flow of river as it has been before installation of the equipment has to be reestablished | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). - Follow OEM’s recommendations for equipment de-commissioning. |
| Inverters, charge controller and other electronic equipment | Installation |  | - Shall only be installed by competent persons. |
|  | Operation |  | - Must be a safe working environment. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have comprehensive insurance policy to adequately cover facilities, employees and third parties. |
|  | Decommissioning & Disposal | - Shall be disposed of in an environmentally friendly way. Waste must be collected and brought to a recycling centre. |  |
| Switch Gear and Distribution Box Conductors Transformers | Transport & Storage |  | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). |
|  | Installation | - Select a site where the risk of flooding is low. - Use fire proved material. - Make provision for oil containment where applicable. | - Installation only by an electrician - Wear all applicable safety gear like safety shoes and safety gloves. - Comply with the Distribution Code. |
|  | Operation | - Care must be taken to prevent oil leakage from transformers and other switchgears. | - Shall be a safe working environment. - Wear appropriate safety gear. - Shall only be operated by trained electricians. - Comply with the Distribution Code. - A comprehensive occupational safety program, including training and site orientation. - Competent persons to effectively respond to emergency events. - Keep a record of all occupational injuries and illnesses and divulge these records to the Commission or the body on demand for examination and analysis. - Functional risk management process. - Should have comprehensive insurance policy to adequately cover facilities, employees and third parties. |
|  | Decommissioning & Disposal | - Upon decommissioning, all foundations shall be removed and the site restored, as far as practicable, to its original condition. - Transformer may contain hazardous and polluting material which must be brought to a collection point where it can be treated. | - Use appropriate lifting and installation equipment due to the weight of the equipment (e.g., crane). - Appropriate protective equipment should be used by persons involved in the de-commissioning exercise. - Access to project/de-commissioning site must be restricted to only persons involved in the decommissioning exercise. |

I, _______________ (name), representative of Mini-Grid Developer/Operator name: ________________________ Mini-Grid Developer/Operator address: ________________________________________________________________Company registration number: _____________________________ am applying for a Permit and therefore warrant that I will comply with the above-mentioned environmental, health and safety guidelines.

In the event of any failure to comply with these guidelines, I am conscious that the competent authorities can impose fines according to the gravity of the infraction.

Date:

Company Representative